Azul Fuel Waste Reduction Project

Technical Report on Fleet Reassignment Screening

Eduardo Souza

Robert Sanabria

Burak Agar

Alexandre Machado

Team 5 | AE 398 Global Experience Brazil | May 2026

Observed operating sample: 2024-01-01 to 2025-11-15

# 1. Abstract and Research Contribution

This report presents a technical screening study of fuel waste reduction opportunities in Azul's mixed-fleet operation. The analysis is organized around the logic implemented in Main Fleet Analysis.ipynb: first, the observed fleet and network are characterized; second, the relationships among distance, trip time, take-off weight, and fuel burn are examined; third, aircraft-specific efficient operating windows are defined; and fourth, route-level interchangeability is tested only where aircraft families overlap in observed efficient operations.

The central empirical result is narrow and operationally interpretable. Among the tested substitution directions, E2-for-E1 is the clearest actionable opportunity because it remains fuel-positive after approximate seat supply is preserved. The final output of this report is therefore a route-priority list for validation, not a complete fleet optimizer and not an automatic replacement rule.

Table 1. Core research result.

| Metric | Value | Interpretation |
| --- | --- | --- |
| Cleaned flight records | 288,107 | Analytical sample after validity filters. |
| Directional routes | 419 | Origin-destination route universe in the cleaned sample. |
| Airports | 113 | Distinct stations appearing as origin or destination. |
| E2-for-E1 priority routes | 7 | Route-level validation queue from sweet-spot overlap and capacity screening. |
| Observed E1 sweet-spot flights | 641 | Observed E1 flight equivalents used for the route-priority estimate. |
| Median equal-seat fuel saving | 26.8% | Median modeled saving after approximate seat supply is held constant. |
| Modeled fuel saving | 481,443 kg | Gross modeled fuel saving over equivalent observed seat supply, not annualized. |

Source: Authors.

# 2. Introduction and Research Question

Fuel burn in airline operations is not a property of aircraft type alone. It is a joint outcome of aircraft family, stage length, trip time, payload, take-off weight, route structure, and departure frequency. For an airline operating a mixed fleet, this creates a methodological problem: aircraft cannot be ranked by raw fuel burn unless they are compared under similar operating conditions.

The research question is: how can Azul identify route-level aircraft reassignment candidates that reduce avoidable fuel burn while remaining inside the observed operating envelope of the candidate aircraft families?

The study treats fuel waste as the difference between observed fuel burn and the modeled fuel burn of a feasible alternative aircraft assignment under comparable mission conditions. The word feasible is used in a screening sense: the model tests observed route overlap and approximate seat supply, but it does not model tail rotations, crew availability, maintenance checks, gate constraints, airport infrastructure, or route economics.

# 3. Data Sources and Analytical Sample

The analysis uses two workbooks and the generated outputs produced from the project notebooks and scripts. Azul_DataSample.xlsx provides flight-level operations. Azul_Project_Data_2.xlsx provides load-evidence averages that are joined to the operating records as proxies for passenger and payload conditions.

Table 2. Data sources used in the analytical workflow.

| File or output | Scope | Analytical role |
| --- | --- | --- |
| Azul_DataSample.xlsx | 288,337 raw flight records | Primary operating data: date, route, flight number, aircraft family, fuel kg, TOW kg, trip time, and distance. |
| Azul_Project_Data_2.xlsx | 8,864 load-evidence records | Passenger, passenger weight, zero-fuel weight, dry operating weight, cargo, and baggage proxies. |
| joined_cleaned_flight_data.csv | 288,107 cleaned and joined records | Main analytical table used for fleet profiling, derived variables, and route-level screens. |
| Main Fleet Analysis.ipynb | Notebook analysis sequence | Exploratory profile, correlations, sweet spots, behavior clustering, route interchangeability, and capacity logic. |

Source: Authors.

Cleaning is intentionally conservative. The goal is not to reshape the dataset but to remove observations whose denominators or physical values make efficiency measures unreliable.

Table 3. Cleaning outcome.

| Cleaning step | Rows removed or count |
| --- | --- |
| Initial rows after required fields | 288,337 |
| Nonpositive fuel removed | 4 |
| Trip time below 600 seconds removed | 29 |
| Implausible fuel per NM removed | 1 |
| Implausible ground speed removed | 196 |
| Final analytical rows | 288,107 |
| Records retained | 99.92% |

Source: Authors.

# 4. Analytical Variables and Derived Metrics

The analysis converts operational fields into comparable measures of mission profile and fuel intensity. The central denominator is nautical miles, because the source data provides GroundDistance [NM] and TripFuelBurnTotal [KG].

Table 4. Variable definitions.

| Variable | Definition | Use in analysis |
| --- | --- | --- |
| Equipment | Aircraft family label: ATR, E1, E2, A320, or A321. | Defines fleet groups, operating regimes, and substitution directions. |
| Route | Directional origin-destination pair. | Defines the route unit for overlap and priority screens. |
| Distance NM | Ground distance in nautical miles. | Stage-length control and denominator for kg/NM. |
| Trip time | TripTimeSec converted to minutes. | Mission-duration control and sweet-spot dimension. |
| Fuel kg | TripFuelBurnTotal [KG]. | Primary fuel outcome. |
| TOW kg | TakeOffWeight [KG]. | Operational proxy for aircraft size, payload, fuel uplift, and mission behavior. |
| Fuel per NM | Fuel kg divided by distance NM. | Primary fuel-intensity measure used to rank efficient observed flights. |
| Passenger proxy | AVG_PassengerTotal joined from load evidence. | Contextual load signal, not true flight-level demand. |
| Seat proxy | Fixed assumed family seats: ATR 70, E1 118, E2 136, A320 174, A321 214. | Equal-seat feasibility screen. |
| Joint sweet spot | Flight inside efficient IQR windows for distance, trip time, and TOW. | Comparable operating sample for route substitution tests. |

Source: Authors.

Fuel intensity is defined as FuelPerNM_i = FuelKg_i / DistanceNM_i. Equal-seat fuel saving is computed by estimating how many flights of the proposed aircraft would be required to provide the same approximate seat supply as the observed worse-aircraft sweet-spot flights.

# 5. Exploratory Fleet and Network Profile

The notebook begins with a descriptive profile because substitution analysis is only meaningful after the fleet's observed mission structure is understood. The cleaned sample confirms a stratified fleet: ATR aircraft concentrate on shorter sectors; E1 and E2 aircraft occupy the regional-jet space; and A320/A321 aircraft operate longer, denser missions.

Table 5. Fleet profile in the cleaned sample.

| Aircraft | Flights | Unique routes | Median distance (NM) | Median trip time (min) | Median TOW (kg) | Median fuel kg/NM |
| --- | --- | --- | --- | --- | --- | --- |
| ATR | 78,452 | 220 | 231.0 | 62 | 21,225 | 2.96 |
| E1 | 44,805 | 269 | 295.6 | 53 | 44,429 | 6.67 |
| E2 | 50,191 | 260 | 382.4 | 64 | 52,745 | 4.98 |
| A320 | 100,865 | 294 | 889.2 | 128 | 66,644 | 5.38 |
| A321 | 13,794 | 96 | 1,170.4 | 167 | 80,116 | 6.01 |

Source: Authors.

This profile explains why raw fleet averages cannot support direct aircraft ranking. An A321 burns more fuel than an ATR because it is assigned to a different mission class. The relevant analytical comparison is therefore not global fleet average versus global fleet average; it is aircraft performance inside an overlapping and operationally plausible route-mission space.

# 6. Correlation Structure and Mission Drivers

The correlation stage in the notebook establishes the basic functional structure of the data. Fuel burn is strongly related to distance and trip time across all major equipment families. TOW is also relevant, but its relationship with fuel is more dispersed because TOW partly reflects aircraft family and route assignment rather than only payload.

Table 6. Equipment-level correlations with fuel burn.

| Aircraft | Fuel vs TOW | Fuel strip time | Fuel vs distance | Interpretation |
| --- | --- | --- | --- | --- |
| A320 | 0.64 | 0.99 | 0.98 | Fuel burn is dominated by stage length and duration. |
| A321 | 0.72 | 0.96 | 0.94 | Longer missions and heavier operations move together. |
| ATR | 0.33 | 0.98 | 0.98 | Short-haul fuel burn is primarily explained by distance and time. |
| E1 | 0.35 | 0.98 | 0.98 | Distance and time dominate within the regional-jet mission band. |
| E2 | 0.51 | 0.95 | 0.95 | Fuel burn scales with mission length, with moderate TOW association. |

Source: Authors.

The theoretical implication is that aircraft substitution should be tested after controlling for mission conditions. Otherwise, the model would attribute to equipment what may be caused by route length, duration, and operational assignment.

# 7. Operating Regimes by Take-off Weight

The next notebook logic uses take-off weight to construct observed operating regimes. TOW is not a perfect measure of demand, but it is a useful operational proxy because it includes the combined effect of airframe, fuel uplift, passengers, baggage, cargo, and mission planning.

Table 7. Observed TOW regimes.

| Aircraft | Flights | Low TOW (kg) | Median TOW (kg) | High TOW (kg) |
| --- | --- | --- | --- | --- |
| ATR | 78,452 | 18,948 | 21,225 | 22,305 |
| E1 | 44,805 | 40,868 | 44,429 | 46,299 |
| E2 | 50,191 | 49,129 | 52,745 | 55,191 |
| A320 | 100,865 | 61,142 | 66,644 | 70,963 |
| A321 | 13,794 | 72,751 | 80,116 | 84,299 |

Source: Authors.

Figure 1. Observed take-off-weight regimes by aircraft family.

Source: Authors.

The regimes form an ordered mission ladder. This supports the use of within-family efficient windows before cross-family comparison: the analysis must first locate where each aircraft is used efficiently in its own observed operating domain.

# 8. Sweet-Spot Methodology

The sweet-spot methodology is the main methodological bridge between exploratory analysis and substitution screening. Within each aircraft family, flights are ranked by fuel per NM. The best-performing 50 percent define the family's observed efficient operating subset. Interquartile windows are then calculated for distance, trip time, and TOW within that subset.

A flight is classified as a joint sweet-spot observation only if it falls simultaneously inside all three windows. This distributional approach avoids imposing a linear fuel curve and reduces the influence of tail observations.

Table 8. Efficient operating windows from the best-performing 50 percent of flights.

| Aircraft | Best-half flights | Joint sweet flights | Distance IQR (NM) | Trip time IQR (min) | TOW IQR (kg) | Joint median fuel kg/NM |
| --- | --- | --- | --- | --- | --- | --- |
| ATR | 39,228 | 14,574 | 221.0 to 321.0 | 60.0 to 86.0 | 19,770 to 21,828 | 2.87 |
| E1 | 22,403 | 7,632 | 308.3 to 482.3 | 53.0 to 78.0 | 42,289 to 45,681 | 6.33 |
| E2 | 25,096 | 9,039 | 440.5 to 839.6 | 68.0 to 129.0 | 51,130 to 54,603 | 4.67 |
| A320 | 50,433 | 15,701 | 888.6 to 1,187.6 | 126.0 to 168.0 | 64,059 to 68,882 | 5.08 |
| A321 | 6,897 | 1,958 | 1,156.1 to 1,198.6 | 158.0 to 173.0 | 74,915 to 80,178 | 5.81 |

Source: Authors.

# 9. Joint Sweet-Spots and Behavior-Based Fleet Clusters

After sweet-spot construction, the notebook groups aircraft according to observed mission profile, efficient mission profile, fuel intensity, and fuel-distance behavior. This step is theoretical as well as descriptive: it tests whether aircraft families are naturally comparable before any substitution is proposed.

Table 9. Behavior-based fleet group interpretation.

| Group | Aircraft families | Analytical interpretation |
| --- | --- | --- |
| Short-haul turboprop missions | ATR | Distinct short-stage operating curve; not naturally interchangeable with jets without a capacity and frequency penalty. |
| Short to medium regional-jet missions | E1 and E2 | Most relevant substitution space because both families operate in overlapping regional and medium sectors. |
| Medium to long-haul narrowbody missions | A320 and A321 | Comparable narrowbody group, but capacity differences are material. |

Source: Authors.

This clustering supports the final focus on the regional-jet pair. E2 is not treated as universally superior; rather, it is treated as a candidate where the observed E1 and E2 route-mission spaces overlap.

# 10. Route Interchangeability Screen

The route interchangeability screen applies the joint sweet-spot definition at route level. A candidate comparison is retained only when both aircraft families are observed on the same route inside their own sweet spots. This is a conservative design choice: substitutions are tested from observed operating evidence rather than hypothetical aircraft deployment.

Table 10. Fuel-only route interchangeability screen.

| Fuel-only direction | Candidate routes | Worse-aircraft sweet-spot flights | Median kg/NM saving | Interpretation |
| --- | --- | --- | --- | --- |
| A320 replacing A321 | 4 | 1,957 | 14.6% | Fuel-positive before capacity is held constant. |
| ATR replacing E1 | 7 | 459 | 57.5% | Large per-flight fuel signal, but likely capacity constrained. |
| E2 replacing E1 | 7 | 641 | 17.7% | Moderate fuel signal in the regional-jet overlap space. |

Source: Authors.

The fuel-only screen is intentionally not the decision rule. It identifies possible directions for substitution, but it does not yet account for the number of seats supplied by each aircraft family.

# 11. Equal-Seat Capacity Screen

The capacity screen converts fuel-only comparisons into an approximate equal-seat comparison. The method attaches fixed seat proxies to each family and estimates the number of alternative-aircraft flights required to provide the same observed seat supply. This screen is a proxy for capacity feasibility, not a passenger-demand model.

Table 11. Capacity-screen result by substitution direction.

| Substitution | Routes | Median extra flights | Median equal-seat saving | Capacity-screen conclusion |
| --- | --- | --- | --- | --- |
| E2 replacing E1 | 7 | 0.0% | 26.8% | Retained. Fuel-positive and no additional frequency required under the seat proxy. |
| ATR replacing E1 | 7 | 68.6% | 33.5% | Not retained. Requires substantial additional frequency to preserve seat supply. |
| A320 replacing A321 | 4 | 23.0% | -4.6% | Not retained. Turns fuel-negative once approximate seat supply is held constant. |

Source: Authors.

The E2-for-E1 result survives because the assumed E2 seat count exceeds the E1 seat count while the observed E2 fuel intensity remains lower in the retained overlap routes. This makes E2-for-E1 the only tested direction that passes both fuel and capacity screens.

# 12. Route Priorities

The report terminates at the route-priority stage. The seven routes below are the proposed validation queue for Azul's technical and scheduling review. They are not automatic reassignment instructions, and they should be evaluated with tail rotations, crew, maintenance, airport, payload, and economic constraints before implementation.

Table 12. E2-for-E1 route validation priorities.

| Route | Observed E1 sweet flights | Median distance (NM) | Median equal-seat saving | Kg per E1 flight equivalent | Modeled kg saved |
| --- | --- | --- | --- | --- | --- |
| CGR -> VCP | 196 | 459.1 | 27.6% | 719 | 140,938 |
| BSB -> VCP | 155 | 463.2 | 30.3% | 825 | 127,893 |
| VCP -> BSB | 160 | 451.2 | 28.1% | 753 | 120,441 |
| GYN -> VCP | 66 | 445.3 | 26.8% | 712 | 46,979 |
| VCP -> IGU | 21 | 456.0 | 26.5% | 728 | 15,279 |
| VCP -> CGR | 20 | 474.4 | 26.6% | 758 | 15,152 |
| IOS -> CNF | 23 | 439.2 | 24.5% | 642 | 14,761 |
| Total / median | 641 | - | 26.8% | 751 | 481,443 |

Source: Authors.

The correct interpretation is prioritization under uncertainty. The route list identifies where E2 allocation is expected to have the strongest fuel benefit within the observed regional-jet overlap space. It does not replace Azul's operational planning systems and does not model network-wide fleet assignment.